Guide Specification - SECTION 08 11 16  ALUMINUM DOORS and FRAMES

WAUSAU 5250i-XLT CROSSTRAK™ SERIES Sliding Glass Doors

[NOTE: Instructional and advisory language in grey italic font. A size limitation chart is provided at the end of this Guide Specification.]

PART 1 GENERAL

  - SUMMARY

- Section Includes:

  - All exterior Architectural Performance Class (AW) sliding glass doors furnished and installed as shown on drawings, specified in this section and designated in AAMA/WDMA/CSA 101/I.S.2/A440 North American Fenestration Standard (NAFS).

  - All labor, materials, tools, equipment and services needed to furnish and install AW Class sliding glass doors.

  - Components furnished with installed sliding glass doors.

  - Installation accessories furnished and installed.

  - REFERENCE

- Refer to NAFS for a complete list of references and industry standards.

1.03 SYSTEM DESCRIPTION AND PERFORMANCE REQUIREMENTS

A. 	 Design Wind Loads - Allowable Stress Design (ASD)

  - The design wind pressure for the project will be:  (Specify)

    - ___ psf positive and negative; ___ psf negative at corner zones

    - Per wind pressure diagram, (or)

    - Per local building codes

[NOTE: Determination of Design Load(s) is the sole responsibility of the building's Engineer of Record, considering Code interpretation issues and/or prescriptive requirements not included in Contract Documents.  WAUSAU is not responsible for determining Design Loads and strongly recommends that ASD Design Loads (in psf or Pa) specific to all relevant areas of the building be provided by the Specifier.]

  - All structural components, including meeting rails, mullions and anchors shall be designed accordingly, complying with deflection and stress requirements of Paragraph 1.03.B.

- Air, Water and Structural Performance Requirements

  - When tested in accordance with cited test procedures, sliding glass doors shall meet or exceed the following performance criteria, as well as those indicated in NAFS for Architectural AW Performance Class sliding glass doors, Performance Grade 100 (AW100) (See Specifier Note below) unless otherwise noted herein.

    - Test units shall not be smaller in either width or height than the “Gateway Test Size” specified in NAFS for AW Performance Class.

    - “Downsize” testing shall not be permitted.

    - Test units shall employ manufacturer’s standard sealing, lock spacing and anchorage.

(Specifier Note:  Units large than AAMA Gateway Test Size will be rated for Performance Grades less than 100, but not less than the AW Performance Class minimum of AW40. Units provided with accessible sill thresholds will be rated as AW-40.)

  - Air Test Performance Requirements

    - Air infiltration maximum 0.3 cfm per square foot at 6.24 psf pressure differential when tested in accordance with ASTM E283.

  - Water Test Performance Requirements

    - Operable panel at interior (low profile sill unavailable): No uncontrolled water leakage at 12.0 psf static pressure differential utilizing sill starter as detailed, with water application rate of 5 gallons/hr.sqft when tested in accordance with ASTM E331.

    - Operable panel at exterior (low profile sill available): No uncontrolled water leakage at 12.0 psf static pressure differential utilizing sill starter as detailed, or 8.0 psf with low profile accessible sills, with water application rate of 5 gallons/hr.sqft when tested in accordance with ASTM E331.

(Specifier Note:  Compliance with Fair Housing Act accessibility requirements for threshold height impacts water penetration resistance performance.  The Wausau CrossTrak Series is not compliant with operating force requirements of ICC/ANSI A117.1 unless equipped with special motorized operators provided by others. Check local code and contact a Wausau market manager for more information.)

  - Structural Test Performance Requirements

    - Uniform Load Deflection Test

      - No deflection of any unsupported span “L” of test unit (framing rails, muntins, mullions, etc.) in excess of L/175 at both a positive and negative load of 100 psf (design test pressure) (See Specifier Note above) when tested in accordance with ASTM E330.

    - Uniform Load Structural Test

      - Unit to be tested at 1.5 x design test pressure, both positive and negative, acting normal to plane of wall in accordance with ASTM E330.

      - No glass breakage; permanent damage to fasteners, hardware parts, or anchors; damage to make sliding glass doors inoperable; or permanent deformation of any main frame or ventilator member in excess of 0.2% of its clear span.

- Life Cycle Testing

  - When tested in accordance with AAMA 910-16, there is to be no damage to fasteners, hardware parts, support arms, activating mechanisms or any other damage that would cause the window to be inoperable at the conclusion of testing.

    - Air infiltration and water resistance tests shall meet the primary performance requirements specified after completion of 4000 operational cycles plus thermal cycling.

    - Testing to previous, less stringent versions of AAMA 910 shall not be acceptable.

- Condensation Resistance and Thermal Transmittance Performance Requirements

(Wausau recommends specifying U-Factor and SHGC requirements by citing the values used in the default, prescriptive or performance compliance path option employed during permitting. Coordinate with 08 80 00 Glazing.)

  - Perform thermal tests in accordance with NFRC and/or AAMA test methods, or provide finite element computer thermal modeling and calculations per NFRC, AAMA and/or manufacturer’s proprietary software as deemed acceptable by the design team. To help ensure proper interpretation, all representations of thermal and solar-optical performance shall be qualified explicitly for the glass type, size, configuration, installation accessories and substrate conditions used in their determination.

    - Thermal Transmittance (U-Factor) for the overall window area shall be less than or equal to _____ BTU/hr-ft2-F.

    - Solar Heat Gain Coefficient (SHGC) for the overall window area shall not exceed ____.

    - Condensation Resistance Factor (CRF) requirements:  CRF minimum ___ (Frame) and CRF minimum ___ (Glass).

(Visit www.fgiaonline.org to access the AAMA online CRF Selection Tool. CRF varies with configuration, frame depth, and glass type. Refer to AAMA 515 for recommendations on condensation resistance specifications for critical applications such as hospitals.)

[DISCLAIMER:  Condensation on interior surfaces of installed windows may be affected by many variables, including, but not limited to, component thermal performance, thermal mass of surrounding materials, interior trim coverage and air flow conditions, weather and mechanical system design.  Since many of these variables are outside of our control, WAUSAU makes no representations or warranties against the formation of condensation. Testing is conducted on pre-defined configurations under controlled and steady-state laboratory conditions, as specified above.]

- Acoustic Performance Requirements

  - Perform acoustical tests in accordance with ASTM E90 and ASTM E1425 on the glass type(s) specified in 08 80 00, rigidly supported in aluminum framing of the same product type.

  - “Glass-only” test results shall not be acceptable.

  - Sound Transmission Class (STC) shall not be less than ___. (Specify STC 31 for 1” insulating glass with makeup of ¼” glass – ½” air space – ¼” glass. Coordinate with 08 80 00 Glazing.)

  - Outdoor-Indoor Transmission Class (OITC) shall not be less than  ____. (Specify OITC 25 for 1” insulating glass with makeup of ¼” glass – ½” air space – ¼” glass. Coordinate with 08 80 00 Glazing.)

- Sustainable Design Requirements

  - The products provided under this section may affect LEED® certification for the project.  Provide available documentation relevant to the project in accordance with USGBC’s “LEED® v4 for Building Design and Construction,” verifying that the components, processes and/or assemblies specified herein conform to the following requirements.  (Select only those Credits needed for the level of certification sought. Modify criteria as necessary for other LEED® programs or versions applicable to specific projects.)

      - Energy and Atmosphere (EA): EA Prerequisite “Minimum Energy Performance,” and EA Credit “Optimize Energy Performance”

        - Area-weighted overall U-Factor not to exceed that specified in 1.03.

        - Area-weighted center-of-glass or overall SHGC less than or equal to that specified in 1.03.

      - Material and Resources (MR): MR Credit “Building Product Disclosure and Optimization – Environmental Product Declarations, Option 1. Environmental Product Declaration (EPD)”

        - Provide all relevant component suppliers’ industry-wide or product-specific EPDs as available.

      - Material and Resources (MR): MR Credit “Building Product Disclosure and Optimization – Sourcing of Raw Materials, Option 2 “Leadership Extraction Practices - Recycled Content.”

        - Provide sliding glass door assemblies (aluminum framing, glass and other components) containing no less than ___% (Specify up to 15%) combined recycled content by assembly weight.

        - Combined content to be calculated as post-consumer plus one-half pre-consumer recycled content by weight.

        - Report pre- and post-consumer recycled content separately.

        - All recycled secondary aluminum billet must meet Aluminum Association content requirements for the alloy used.

[NOTE: Secondary aluminum billet contains a high percentage of pre-consumer recycled content, however, glass comprises the majority of the weight of a glazed window assembly.]

    - Material and Resources (MR): MR Credit “Building Product Disclosure and Optimization – Material Ingredients.”

      - Provide all relevant component suppliers’ disclosures, including manufacturers’ inventory of commonly used materials and ingredients where available.

    - EQ  Low-Emitting Materials – Adhesives and Sealants

      - All interior primers, structural glazing adhesives and metal-to-metal sealants used on site must meet applicable South Coast Air Quality Management District (SCAQMD) Rule #1168 VOC limits.

    - EQ  Thermal Comfort- Design

      - Provide thermal barrier framing and glazing as specified herein and in 08 80 00 Glass and Glazing.

    - EQ  Daylight and Views

      - Area-weighted overall VT not less than 0.____ (Specify – Coordinate with 08 80 00 Glazing) per NFRC 200.

      - Sliding glass door sizes and configurations as shown on drawings.

  - SUBMITTALS

- General Requirements

  - Provide all submittals in a timely manner to meet the required construction completion schedule.

- Shop Drawings

  - Shop drawings must be prepared wholly by the sliding glass door manufacturer, or a qualified engineering services firm under the guidance of the manufacturer.  Shop drawings for pre-engineered configurations may be prepared by installers upon written consent of the manufacturer.

  - Provide design details along with bid proposals to define system aesthetic and functional characteristics.

  - Provide up to three photocopied sets of shop drawings, including half size details of all necessary conditions.

- Samples

  - Components:  Submit samples of anchors, fasteners, hardware, assembled corner sections and other materials and components as requested by Architect.

  - Finish:  Submit color samples for Architect's approval as requested.

- Test Reports and Calculations

  - Submit certified independent laboratory test reports verifying compliance with all test requirements of 1.03.

  - Submit structural calculations indicating adequacy of all materials furnished under this section, to meet the uniform and structural load requirements as specified in 1.03.

  - QUALITY ASSURANCE

- Qualifications:  Upon request, the sliding glass door manufacturer shall provide written consent for the installation subcontractor to install sliding glass door products to be used on this project.

  - DELIVERY, STORAGE AND HANDLING

- Packing, Shipping, Handling and Unloading

  - Materials will be packed, loaded, shipped, unloaded, stored and protected in accordance with AAMA CW-10.

  - WARRANTY

- Aluminum Sliding Glass Door Warranty

  - Products:  Submit a written warranty, executed by the sliding glass door manufacturer, for a period of ___ (up to 10) years from the date of manufacture, against defective materials or workmanship, including substantial non-compliance with applicable specification requirements and industry standards, which result in premature failure of the sliding glass doors, finish, factory-glazed glass, or parts, outside of normal wear.

[NOTE: Refer to the WAUSAU Standard Limited Warranty for detailed exclusions, qualifications and limitations. Screens and balances are warranted for one year.]

  - In the event that sliding glass doors or components are found defective, manufacturer will repair or provide replacement material without charge at manufacturer’s option.

  - Warranty for all components must be direct from the manufacturer (non pass-through) and non pro-rated for the entire term.  Warranty must be assignable to the non-residential owner, and transferable to subsequent owners though its length.

PART 2 PRODUCTS

2.01 MANUFACTURERS

- Acceptable Manufacturer

  - Drawings and specification are based on:

    - Wausau Window and Wall Systems – 5250i-XLT CrossTrak™ Series Sliding Glass Doors.

    - Base bid will be Wausau Window and Wall Systems.

  - Substitutions

    - Other manufacturers’ products that meet or exceed specified design requirements may be considered.  Submit the following information with request for substitutions at least ten (10) working days prior to bid date.

      - Test reports specified in 1.03.

      - Full proposal details and samples specified in 1.04.

      - Copy of manufacturer's warranty specified in 1.07.

      - Proof of at least 10 years experience in the design and fabrication of AW Performance Class sliding glass doors.

      - Other information as requested for evaluation

  - Substitute products not pre-approved by the Architect via addenda will not be considered.

  - Clear preference will be given to products produced in LEED®-certified manufacturing  facilities.

2.02 MATERIALS

- Aluminum Framing Members

  - Extruded aluminum billet, 6063-T5 or T6 alloy for primary non-radius components; 6063-T5 or T6, 6005-T5, 6105-T5 or 6061-T6 for anchor components; all meeting the requirements of ASTM B221.

  - Aluminum sheet alloy 5005-H32 (for anodic finishing), or alloy 3003-H14 (for painted or unfinished sheet) meeting the requirements of ASTM B209.

  - Principal sliding glass door frame members will be a minimum 0.125" in nominal thickness.

  - Extruded or formed trim components will be a minimum 0.060" in nominal thickness.

  - Frame depth 5-7/8" minimum.

  - Sills shall be sloped. Sill must allow for drainage to the exterior.

  - Door stile extrusions must be tubular.

    - (Select operation) Interior (Exterior) door panel(s) shall be operable; exterior (interior) panel(s) fixed in place.

(Specifier Note: Interior operable panels are used with Juliet balcony railings, and sill members are not designed for ease of access.  Exterior operable panels are used at balconies or terraces, and sill members can be designed for ease of access at a  lower water penetration resistance test pressure).

    - Door panels shall be placed to provide (Select configuration) half-slide (center-slide) (end-slide) (center-open) operation as indicated on architectural drawings.

    - All sliding glass doors shall be two-track.

  - Frames to be designed such that each fixed and operable module is self-mulling (stacking) for rigidity and load transfer, as well as incorporation of integral single-frame transom and spandrel lites.

2.03 COMPONENTS

- Hardware (Select from options as noted)

  - All attachment fasteners to be 300 Series stainless steel.

    - Other steel components shall be protected by corrosion resistant coatings meeting the requirements of AAMA 907.

  - Extruded aluminum components 6063-T5 or -T6.

  - Locking handles, bases and strikes to be die cast, white bronze, aluminum, or stainless steel in manufacturer’s standard surface finish.

  - Thermo-plastic or thermo-set plastic caps, housings and other components to be injection-molded nylon, extruded PVC, or other suitable compound.

  - Hardware to be occupant-operated and include: (Select from appropriate options listed in the WAUSAU Hardware Selection Guide: Conventional sliding glass doors: Coated steel roller assemblies, stainless steel roller assemblies, heavy-duty Truth Mammoth™ roller assemblies, white bronze pull latches, stainless steel mortise latches, limit stops, anti-take-out blocks. Euro “lift and slide” doors: Coated steel roller housings with nylon rollers, heavy-duty coated steel roller housings with nylon rollers, die cast operating handles and thumb turns, coated steel gear box housings, polyamide nylon slide bars, stainless steel locks and strikes, anti-take-out blocks.)

- Sealants

  - All sealants shall comply with applicable provisions of AAMA 800 and/or Federal Specifications FS-TT-001 and 002 Series.

  - Frame joinery sealants shall be suitable for application specified and as tested and approved by sliding glass door manufacturer.

- Glass

  - Provide in accordance with Section 08 80 00.

  - Sealed insulating glass shall be tested and certified in accordance with ASTM E2190.

- Glazing

  - Provide in general accordance with Section 08 80 00.

  - Glazing method shall be in general accordance with the GANA Glazing Manual for specified glass type, or as tested and approved by the manufacturer.

  - Only bead glazing is acceptable, allowing for glass replacement without disassembly of operating or fixed door panels.

- Glazing Materials

  - Setting Blocks/Edge Blocking:  Provide in sizes and locations recommended by GANA Glazing Manual. Setting blocks used in conjunction with soft-coat low-e glass shall be silicone.

  - Back-bedding tapes, expanded cellular glazing tapes, toe beads, heel beads and cap beads shall meet the requirements of applicable specifications cited in AAMA 800.

  - Glazing gaskets shall be non-shrinking, weather-resistant, and compatible with all materials in contact.

  - Structural silicone sealant where used shall meet the requirements of ASTM C1184.

  - Spacer tape in continuous contact with structural silicone shall be tested for compatibility and approved by the sealant manufacturer for the intended application.

  - Gaskets in continuous contact with structural silicone shall be extruded silicone or compatible material.

- Steel Components

  - Provide steel reinforcements as necessary to meet the performance requirements of 1.03.

  - Concealed steel anchors and reinforcing shall be factory painted after fabrication with TGIC powder coating, or rust-inhibitive primer complying with Federal Specification TT-P-645B.

- Muntins: (Optional)

  - Provide muntin grids as shown on architectural drawings.

  - Finish to match sliding glass door frames.

- Receptors:

  - Provide extruded aluminum receptors to receive sliding glass doors, as shown on architectural drawings.

  - Finish to match sliding glass door frames.

- Insect Screens: (Optional)

  - Provide half screens with jamb latches. Screens shall be located at the (Select one) exterior (interior) side of the operable door panel.

  - Tubular extruded aluminum frames shall meet the requirements of ANSI/SMA 1004.

  - Screen frame finish to match sliding glass door frames.

  - Aluminum cloth shall comply with GSA-FS-RR-W-365 and USDC-CS-138 with 18 x 16 mesh.

  - Cloth mesh color shall be ________________. (Specify bright mesh or charcoal mesh)

- Juliet Balconies:

  - Provide extruded aluminum framing to receive (Select one) factory-glazed (factory-assembled aluminum) Juliet balcony railings as shown on architectural drawings.

  - Finish to match sliding glass door frames.

  - Juliet balcony railings are used only with operable panels at the interior.

2.04 FABRICATION

- General:

  - Finish, fabricate, factory-assemble and factory-glaze operable and fixed panels under the responsibility of one manufacturer.

  - No bolts, screws or fastenings shall impair independent frame movement, or bridge the thermal barrier, unless such bridging was also present in thermal test units and thermal models.

  - Fabricate to allow for thermal movement of materials when subjected to a temperature differential from -30 F to +180 F.

- Frames:

  - Cope and mechanically fasten each corner, then seal weather tight.

  - Make provisions for continuity of frame joinery seals at extrusion webs.

- Door Stiles:

  - Miter all corners and mechanically stake over solid extruded aluminum corner blocks, set and sealed in epoxy, leaving hairline joinery, then sealed weather tight.

  - Make provisions for continuity of door stile joinery seals at extrusion webs.

- Glass Drainage: (field glazed units only)

  - Provision shall be made to insure that water will not accumulate and remain in contact with the perimeter area of sealed insulating glass.

  - Hardware: (Select from options as noted.)

    - Rollers:

      - Operable panel sill must be equipped with adjustable roller housings with tandem ball-bearing rollers.

      - (Select one) Conventional sliding door rollers shall be easily adjustable, without removal of panel, to provide proper operation, fit and seal. (Lift and slide rollers may be non-adjustable for door sizes exceeding 440 lbs Special structural support and stability provisions are necessary.)

      - Roller edges shall be concave to ride on 300 Series stainless steel track covers, engaged in black polyamide sill tracks installed in frame assemblies.

    - Roller housings must be replaceable without the use of special tools.

  - Locks:

    - Operating door panel must be equipped with latches at the jamb stile to securely lock rolling panel in closed position.

    - Provide an integral extruded radius-edge pull handle at inner and/or outer sash meeting rail interlock.

    - Secure the operable panel with anti-lift clips, placed in an inconspicuous location.

  - Operating Handle Options: (Select appropriate combination from the WAUSAU Hardware Selection Guide.)

    - Provide a clear-coated white bronze pull handle at the jamb stile. (Conventional sliding glass doors)

    - Provide die cast handles in color selected from manufacturer’s standard offering, to operate a coated steel gear box, polyamide or extruded aluminum slide bars, and coated steel corner drives, which in turn operate lifting roller assemblies, minimizing sliding friction at top and bottom weather-seals during operation.  (Euro “lift and slide” doors)

E. 	Thermal Break Construction

  - Continuous extruded black polyamide with 25% glass fiber reinforcing, mechanically crimped into cross-knurled cavities.

  - Minimum thermal barrier width 14 mm.

  - Quality assurance records must be maintained and available as requested.

F. 	Weather-stripping:

  - Bulb-, fin- and/or sweep-type neoprene, EPDM, dual-durometer PVC, polypropylene, TPE, “silicoated” woven pile weather-strip with MylarTM side or center fins, or other suitable material as tested and approved by the window manufacturer.

  - Miter, crowd, stake or join at corners. Provide drainage to exterior as necessary.

2.05 FINISHES

- Finish of Aluminum Components

  - Finish of all exposed areas of aluminum windows and components shall be done in accordance with the appropriate AAMA Voluntary Guide Specification shown (Select from below).

Designation			Description			Standard 				Color

AAM10C21A41			Clear Class I 			AAMA 611				Clear

Eco-friendly etch

AAM10C21A44			Electrolytically			AAMA 611				Champagne, Light Bronze, Medium Bronze

Deposited Class I								Dark Bronze, Extra Dark Bronze, Black, or				Eco-friendly etch								Copper Anodize

Interior Paint			50% PVDF,				AAMA 2603				____________ (As selected by Architect)

Enamel, Acrylic									Suitable for INTERIOR finishes

or Polyester

Exterior Paint			70% PVDF				AAMA 2605				____________ (As selected by Architect)

Suitable for INTERIOR or EXTERIOR finishes

[NOTE: For detailed specifications of other surface treatments and finishes available, as well as sustainable finishing process specification upgrades contact Linetec or Wausau Window and Wall Systems. Color selection is recommended for the most competitive bids.]

PART 3 EXECUTION

3.01 EXAMINATION

- Site Verification of Conditions

  - Verify that building substrates permit installation of sliding glass doors according to the manufacturer's instructions, approved shop drawings, calculations and contract documents.

  - Do not install sliding glass doors until unsatisfactory conditions are corrected.

3.02 INSTALLATION

- Erection of Aluminum Sliding Glass Doors

  - Install sliding glass doors with skilled workers in accordance with approved shop drawings, installation instructions, specifications, and the AAMA Commercial Window and Door Installation Manual.

  - Sliding glass doors must be installed and remain plumb, square and level for proper weathering and operation.

  - Aluminum that is not organically coated shall be insulated from direct contact with steel, masonry, concrete or other dissimilar metals by bituminous paint, rust-inhibiting primer, non-conductive shims or other suitable insulating material.

[DISCLAIMER:  Wausau Window and Wall Systems takes no responsibility for product selection or application, including, but not limited to, compliance with building codes, safety codes, laws, merchantability or fitness for a particular purpose; and further disclaims all liability for the use, in whole or in part, of these Guide Specifications in preparation of project specifications and/or other documents.  Guide Specifications are subject to change at any time, without notice, and at Wausau’s sole discretion.]

“CrossTrak” is a trademark of Apogee Wausau Group, Inc. All rights reserved. ©2021 Apogee Wausau Group, Inc.